CORPORATE CARBON FOOTPRINT REPORT

TS EN ISO 14064-1:2019

Greenhouse gases — Part 1: Specification with guidance at the organization level for quantification and reporting of greenhouse gas emissions and removals

2025

ERTE KOZMETİK SANAYİ VE TİCARET A.Ş.

| NAME OF THE ORGANIZATION | ERTE KOZMETİK SANAYİ VE TİCARET A.Ş. |
| --- | --- |
| ADDRESS | Cihangir, Güvercin Caddesi No:1, 34310 Avcılar/İstanbul |
| TELEPHONE | +90 (212) 875 81 82 |
| WEBSITE | https://ertecosmetics.com |
| ACTIVITIES | Development and production of perfumes, house fragrances, bath and body care products, and skin care and hair care products |
| PROJECT DESCRIPTION | Calculation of Carbon Footprint Values within the scope of TS EN ISO 14064-1:2019 and GHG Protocol |
| PERSON IN CHARGE | Pamuk Durak |
| REPORT DATE | 13.04.2026 |
| REPORT REVISION DATE | - |
| REPORT REVISION NO. | 00 |
| FREQUENCY | Once a year |
| REPORT PERIOD | 01.01.2025-31.12.2025 |
| BASE YEAR | 2022 |

GREENHOUSE GAS EMISSION RESULTS

Greenhouse gas emissions are presented in Table 1 summary chart.

Table 1 Greenhouse Gas Emissions Summary Chart

| Emissions | SF6 | HCFC | CO2 | CH4 | N2O | Total tCO2e |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (tCOıe) | (tCOıe) | (tCOıe) | (tCO2e) | (tCO2e) |  |  |
| Category 1 | 0.00 | 7.41 | 138.86 | 0.10 | 0.84 | 147.22 | 1% |
| Category 2 | 0.00 | 0.00 | 468.74 | 2.04 | 2.70 | 473.47 | 4% |
| Category 3 |  |  |  |  |  | 2,156.54 | 18% |
| Category 4 |  |  |  |  |  | 9,454.25 | 77% |
| Category 5 |  |  |  |  |  | 22.64 | 0% |
| Category 6 |  |  |  |  |  | 0.00 | 0% |
| Total tCO2e |  |  |  |  |  | 12,254.12 | 100% |

Details of distribution of the company’s greenhouse gas emissions by category are shared in Figure 1.

Category 6

Category 5

Category 4

Category 3

Category 2

Category 1

Category 6

Category 5

Category 4

Category 3

Category 2

Category 1

GREENHOUSE GAS EMISSIONS

GREENHOUSE GAS EMISSIONS

Figure 1 Distribution of Greenhouse Gas Emissions

Details of distribution of the company’s direct emissions are shared in Figure 2.

| Direct emissions from stationary combustion sources | Direct process emissions and removals from industrial processes | Direct emissions from mobile combustion sources | Direct emissions from leaking/escaping of greenhouse gas in anthropogenic systems |
| --- | --- | --- | --- |

DIRECT EMISSIONS

DIRECT EMISSIONS

Figure 2 Distribution of Direct Emissions

Details of distribution of the company’s indirect emissions are shared in Figure 3.

DIRECT EMISSIONS

DIRECT EMISSIONS

| Category 2 | Category 3 | Category 4 | Category 5 | Category 6 |
| --- | --- | --- | --- | --- |

Figure 3 Distribution of Indirect Emissions

Details of distribution of transportation-related emissions are shared in Figure 4.

| Emissions from transportation or distribution of goods (entering the organization/upstream) | Emissions from transportation or distribution of goods (leaving the organization/downstream) | Emissions related to business travel and visitors | Emissions from employee commuting |
| --- | --- | --- | --- |

TRANSPORTATION-RELATED INDIRECT EMISSIONS

TRANSPORTATION-RELATED INDIRECT EMISSIONS

Figure 4 Distribution of Transportation-Related Emissions

Details of distribution of indirect greenhouse gas emissions from products used by the organization are shared in Figure 5.

| Emissions from purchased raw materials/finished goods/semi-finished goods, etc., related to the manufacturing of the product | Emissions from capital assets (movable & immovable) | Fuel and energy activities | Emissions from disposal of solid and liquid wastes |
| --- | --- | --- | --- |

INDIRECT GHGE FROM PRODUCTS USED BY THE ORGANIZATION

INDIRECT GHGE FROM PRODUCTS USED BY THE ORGANIZATION

Figure 5 Distribution of Emissions from Products Used by the Organization

Details of distribution of greenhouse gas emissions by subcategory are shared in Table 2.

Table 2 Distribution of Greenhouse Gas Emissions by Subcategory

| Category 1 - Direct GHG Emissions and Removals | HCFC (tCO2e) | SF6 (tCO2e) | CO2 | CH4 | N2O | Emissions (tCO2e) |  | Total (tCO2e) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Direct emissions from stationary combustion sources | 0.00 | 0.00 | 111.20 | 0.06 | 0.07 | 111.34 | 76% | 147.22 |
| Direct process emissions and removals from industrial processes | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0% |  |
| Direct emissions from mobile combustion sources | 0.00 | 0.00 | 27.65 | 0.04 | 0.77 | 28.46 | 19% |  |
| Direct emissions from leaking/escaping of greenhouse gas in anthropogenic systems | 7.41 | 0.00 | 0.01 | 0.00 | 0.00 | 7.42 | 5% |  |
| Category 2 - Indirect GHGE from imported Energy | HCFC (tCO2e) | SF6 (tCO2e) | CO2 | CH4 | N2O | Emissions (tCO2e) |  | Total (tCO2e) |
| Indirect emissions from imported electricity | 0.00 | 0.00 | 468.74 | 2.04 | 2.70 | 473.47 | 100% | 473.47 |
| Category 3 - Indirect GHGE related to Transportation |  |  |  |  |  | Emissions (tCO2e) |  | Total (tCO2e) |
| Emissions from transportation or distribution of goods (entering the organization/upstream) |  |  |  |  |  | 887.49 | 41% | 2,156.54 |
| Emissions from transportation or distribution of goods (leaving the organization/downstream) |  |  |  |  |  | 306.92 | 14% |  |
| Emissions related to business travel and visitors |  |  |  |  |  | 12.24 | 1% |  |
| Emissions from employee commuting |  |  |  |  |  | 949.88 | 44% |  |
| Category 4 - Indirect GHGE from Products Used by the Organization |  |  |  |  |  | Emissions (tCO2e) |  | Total (tCO2e) |
| Emissions from purchased raw materials/finished goods/semi-finished goods, etc., related to the manufacturing of the product |  |  |  |  |  | 9,106.54 | 96% | 9,454.25 |
| Emissions from capital assets (movable & immovable) |  |  |  |  |  | 224.18 | 2% |  |
| Fuel and energy activities |  |  |  |  |  | 112.66 | 1% |  |
| Emissions from disposal of solid and liquid wastes |  |  |  |  |  | 10.88 | 0% |  |
| Category 5 - Indirect GHGE from Use of Our Products |  |  |  |  |  | Emissions (tCO2e) |  | Total (tCO2e) |
| Emissions arising from processing of sold products |  |  |  |  |  | 0.00 | 0% | 22.64 |
| Emissions and removals from use of sold products |  |  |  |  |  | 0.00 | 0% |  |
| Emissions from end-of-life treatment of products |  |  |  |  |  | 22.64 | 100% |  |
| End-of-life treatment of sold products |  |  |  |  |  | 0.00 | 0% |  |
| Downstream leased assets |  |  |  |  |  | 0.00 | 0% |  |
| Franchises |  |  |  |  |  | 0.00 | 0% |  |
| Emissions from investments |  |  |  |  |  | 0.00 | 0% |  |
| Category 6 - Indirect GHGE from Other Sources |  |  |  |  |  | Emissions (tCO2e) |  | Total (tCO2e) |
| Other Indirect Emissions |  |  |  |  |  | 0.00 | 0% | 0.00 |